TIMS-0201

USB Single Axis Stepper Motor Driver

Operating Manual

http://www.wireworkswest.com

Model TIMS-0102

Part No. 910-0102

Published September 2004

INTRODUCTION

WireWorks West’s TIMS-0201 module offers a quick and easy method of controlling a stepper motor. The module is a single axis controller and supports quadrature encoder inputs and both hard and soft limits. There are also 6 pins of general purpose I/O functionality including digital inputs and outputs, an analog input, a timer/counter, and a PWM output.

The TIMS-0201 module includes a USB 1.1/2.0 Full Speed interface for connection to a host PC. A separate 2.1 mm connector accepts power from an external +10 to +40 VDC power supply. A DB15S connector contains the motor control and the GPIO signals.

The TIMS-0201 module firmware combined with the included driver software allows for flexible, accurate, and consistent motion control.

TIMS-0201 Main Features

- USB Interface

- Upgradeable Firmware

- Home, CW Limit, CCW Limit Inputs

- Quadrature Encoder Inputs

- Half and Full Stepping

- General Purpose I/O Pins

- Bipolar Constant Current Motor Drive with Wide Range of Programmable Current Control (5 – 1000 mA)

- Fast Recovery Schottley Commutation Diodes

- Thermal Overload Protection

- Single 10 – 40VDC Power Supply Input up to 2 amps

A complete LabVIEW driver library is provided for easy integration into higher level applications. A low-level Dynamic Link Library (DLL) is also provided for integration with other programming languages.

- TIMS-0201 STEPPER CONTROLLER Module HARDWARE

The TIMS-0201 Stepper Motor Driver module consists of electronic circuitry packaged in a 1.20”-high by 4.50”-wide by 3.25”-deep (including the connector) plastic enclosure. A USB Type-B connector is located on the rear panel and an additional 2.1 mm 10 – 40 VDC power input connector. The front panel contains both a DB-15S connector.

  - Connector Details

    - USB Connector

The TIMS-0201 USB interface is a USB 1.1 and USB 2.0 Full Speed compliant interface, available through a USB Type-B connector on the rear panel. This device, when connected to user circuitry, could draw up to 500 mA. Do not plug the TIMS-0102 into a non-powered hub.

    - Motor Power Connector

A 3-pin 2.1 mm power input connector on the rear panel accepts 10 – 40 VDC for motor power. The TIMS-0201 module uses a maximum on 1 Amp per winding for a total of 2 Amps maximum current for the module.

    - Motor Signal and GPIO Connector

A DB15S connector on the front panel of the TIMS-0201 module contains contains the 6 connections that go to the motor, 6 GPIO pins, 2 ground connections and a +5V (thru 120 Ohms) connection.

DB-15S Pin Descriptions

| Pin No. | Signal | Notes |
| --- | --- | --- |
| 1 | Winding A – Phase B |  |
| 9 | Winding A – Center Tap | Not Connected – Reserved for Future Use |
| 2 | Winding A – Phase A |  |
| 10 | Winding B – Phase B |  |
| 3 | Winding B – Center Tap | Not Connected – Reserved for Future Use |
| 11 | Winding B – Phase A |  |
| 4 | GND |  |
| 12 | GPIO-0 |  |
| 5 | GPIO-1 |  |
| 13 | GPIO-2 |  |
| 6 | GPIO-3 |  |
| 14 | GPIO-4 |  |
| 7 | GPIO-5 |  |
| 15 | +5V thru 120 Ohms |  |
| 8 | GND |  |

Note: the GPIO pin functionality for pins 4,5,6,7,12,13,14,15 is programmable. The options available for each pin is shown in the table below.

  - TIMS-0201 Signals

    - Motor Control Signals

The TIMS-0201 module provides half and full stepping of 2-phase stepper motors. The stepper motor drive circuit utilizes two Unitrode (Texas Instruments) UC3717 integrated circuit devices. Six pins of the DB15S connector are used for the motor control signals and need to be wired directly to the motor. Pins 1,2, and 9 are dedicated to winding A phase B, center tap, and winding A phase A signals. Pins 10, 3 and 11 are the same but for winding B. The power for these signal is provided from the external power connector and the voltage levels and current restrictions are based on the external power supply used.

The TIMS-0201 has the ability to drive a stepper motor in both full and half steps.  The step mode is controlled by firmware in the TIMS-0201 and programmable thru the driver. Please consult product literature for the motor you wish to drive to determine which mode is correct for your application.

[Greg? Is there anything else we can say about these signals?]

    - GPIO Signals

The TIMS-0201 module contains 6 pins of general purpose I/O. Each pins functionality is programmable according to the table shown above. Some of the GPIO pins can be configured to be limit or home switches. The DB15S connector also contains +5V and GND for use in wiring external switches.

      - Digital I/O Signals

The first five GPIO pins (DB15S pins 12,5,13,6, and 14) can be used for digital input or output. These signals are standard TTL level signals. [Greg? Is there anything else we can say about these signals?]

      - Forward and Reverse Limits and Home

Any of the first five GPIO pins (DB15S pins 12,5,13,6, and 14) can be used for limit or home switch signals. These signals are standard TTL level signals and are configurable thru the driver software. When configured as home or limit inputs the voltage will be monitored and a low level input will effect a motion stop. [Greg? Is there anything else we can say about these signals?]

      - Encoder Input Signals

The first two GPIO pins (DB15S pins 12 and 5) can be used for quadrature encoder inputs. These signals are standard TTL level signals. The TIMS-0201 can use these encoder inputs or an internal counter when controlling motion or reporting position and can be enabled thru the driver. [Greg? Is there anything else we can say about these signals?]

      - Timer/Counter Signal

One of the GPIO pins  (DB15S pin 6) can be used as a timer/counter input. This signals is a standard TTL level signal. [Greg? Is there anything else we can say about these signals?]

      - PWM Signal

One of the GPIO pins  (DB15S pin 14) can be used as a PWM Output. This signals is a standard TTL level signal. The period and duty cycle of this PWM output is programmable thru the driver software. [Greg? Is there anything else we can say about these signals?]

      - Analog Input Signal

One of the GPIO pins  (DB15S pin 7) can be used as a 10-bit analog input. This input can be configured to use the USB 5VDC as a reference or a built-in 4.096 VDC reference. [Greg? Is there anything else we can say about these signals?]

    - +5V and GND Signals

Pin 15 of the DB15S connector on the TIMS-0201 module is connected to the USB bus +5V supply thru an 120 Ohm resistor. Pin 8 is connected to USB Ground.

- 1.3 Controller Functionality in detail

(MAYBE MOVE THIS STUFF TO THE SOFTWARE SECTION)

- 1.3.1 Stepping Vs Half Stepping

The 0201 has the ability to drive a stepper motor in both full and half steps.  Please consult product literature for the motor you wish to drive to determine which mode is correct for your application.

- 1.3.2Encoder Vs Counter

All stepper motors can be driven by specifying the number of steps to move.  However, in this mode there is no feedback to the controller.  It is entirely possible that the motor will get out of sync with the controller.  For example, the controller may output the electrical signals that should cause the motor to move 100 steps, but due to the load on the motor, the motor actually moves only 96 steps.  This is called losing steps.

To prevent this, one can use a motor equipped with an encoder.  The encoder is a feedback device that outputs signals back to the motor controller as the axis moves.  The controller keeps track of these signals and is able to use this information to determine actual position.  In order to use this feature of the 0201, you must first have a motor equipped with an encoder.  You must then configure the 0201 General Purpose IO pins (GPIO) to utilize the encoder, and then physical make the connections.

See section Error: Reference source not found for details about configuring and connecting the encoder inputs.

The 0201 is capable of control using either motor counts or encoder counts.

- 1.3.3Home and Hard Limits

When a motor controller is first powered on, it has no way of knowing what state the motor and physical system is in.  In order to determine this, the system is usually carefully and slowly driven to a safe, known position, and then the step (and possible encoder) counters are set back to zero.  This is known as the Home position.

The home position is usually specified by tripping an optical sensor with a flag.  The signal from the sensor is connected to the HOME input on the controller.  When homing, the 0201 will move at a pre-configured seek speed toward the home sensor.  This seek speed is usually much less than normal operating speed.  When the controller sees that the optical sensor has been tripped, it will stop the motor.  At this point the counters are set to zero, and normal movement can take place.

Additionally, most physical systems have travel limits.  That is, the system is physically unable to move past certain points.  These points are known as hard stops, and they may be designed with the intent that there be physical contact with moving parts, or designed in such a way that the moving parts are not ever supposed to touch the hard stops.

The Hard Limits sensors were designed for this purpose.  Physically these are optical sensors like the home sensor, set up at the forward (FOR) and reverse (REV) limits of travel.  When properly configured, the 0201 controller will stop movement of the motor when it detects that one of these limits has been reached.  Only movement in the opposite direction will be allowed.

The 0201 is configured such that if a hard travel limit is reached while seeking home, the motor will be reversed until home is reached.

- 1.3.4Soft Limits

The 0201 is also capable of using Soft Limits.  Soft limits are programmable limits that act like hard limits, except that they may be changed programmatically and enabled/disabled.  The 0201 is programmed such that if soft limits are enabled and the controller detects that the motor is beyond one of the limits (min or max), then movement will stop and further movement will only be allowed in the opposite direction.

- 1.3.5PWM Generation

- General Software Description

The TIMS-0201 module is delivered with a full LabVIEW instrument driver is available for use in custom applications written using LabVIEW. A low level DLL interface is also available with a fully documented API for use in other applications.

This document is limited to the LabVIEW Driver library. Contact WireWorks West for a copy of the TIMS Packet I/O Developers Manual for more information on the lower level interfaces available for integration with other programming languages.

Figure 1 – Software Organization

The TIMS-0201 module utilizes USB interface chips from FTDI. The FTD2XX.DLL is the lowest level software component on the Host PC and all communications to the TIMS-0201 goes through this DLL.

A complete LabVIEW Driver Library is provided for those who use LabVIEW, which eliminates the need to communicate directly with the DLL.

The complete low level packet communication protocol is also provided later in this section so that C, C++, Visual Basic, and other programmers can communicate with the TIMS-0201 directly through the FTD2XX.DLL.

Each of these software layers is described in detail in the following sections.

- Software Installation

  - Preparing For Installation

Before starting the installation process, you will need the following:

Hardware:

- A PC with a USB 1.1 or 2.0 port

- The TIMS-0201 Module

- A USB cable

Software:

- TIMS-0201 Software Installation CD

- National Instruments LabVIEW 7.0 Software

Documentation:

- TIMS-0201 Manual (this document)

  - Installation

    - Installing the TIMS-0201 USB Drivers

- Place the Installation CD into the CD-ROM drive.

- Connect the TIMS-0201 to your computer.

- Windows should detect the new device and pop up the following dialog:
(The dialog shown below is from Windows XP Service Pack 2; Windows XP, XP SP1 and Windows 2000 will have similar dialogs).

- Select the option “No, not this time” to have Windows Update not search for the device.

- Next, you should choose “Install the software automatically (Recommended).”

- Windows should find the FTDI driver on the CD. Two files may be listed; select either one:

If Windows fails to find the FTD2XX.INF file, you can specify it manually. It is on the CD in the \install\usb folder.

- If you see the following dialog, select “Continue Anyway”:

- Windows should finish installing your driver.

    - Verifying Installation

From the Windows desktop, right click on “My Computer” and select “Properties”. From the Hardware tab press the Device Manager button to open the device manager. Expand the Universal serial Bus controller item. You should see an entry for the FTDI device.

NOTE: There will be no entry in this list if the device is not connected to the PC.

    - Installing the TIMS-0201 LabVIEW Drivers and Code

The TIMS-0201 LabVIEW drivers are located in the \code folder on the installation CD.

- Copy the entire “TIMS-0201” folder to wherever you would like the drivers to reside on your hard disk. (We recommend using your VI.LIB folder within the LabVIEW directory.)

- Remove the CD from your drive.

- Inside the folder, you will also find the file “dir.mnu”. This file may be used to insert a TIMS-0201 submenu into your LabVIEW Palettes. (Refer to LabVIEW documentation for information about menu files.)

- The file “TIMS-0201_Tree.VI” contains links to all the VIs in the library, and is a good starting place to find examples.

- TIMS-0102 LabVIEW Instrument Drivers

The LabVIEW drivers are installed, by default, into aTIMS-0201 directory. The driver library can be broken down into four functional groups:

- I2C Basic Functions

- I2C Scripting Functions

- SPI Functions

- General Functions

The TIMS0102_Tree.vi (installed to C:\WireWorksWest\TIMS-0102\TIMS0102_Tree.vi) diagram contains all the top-level functions needed for most applications as well as several examples.

  - TIMS-0201 Communications Port

A communications port must be opened through USB to the TIMS-0201 before any other driver function call. It is imperative that the port also be closed at the conclusion of a communications session. Leaving a port open will cause some function calls to error and, will also disallow any other communications ports to be opened.

The driver VI’s TIMS Port Open.vi and TIMS Port Close.vi are used for opening and closing communications ports to the TIMS-0201 module.

The TIMS Open Port.vi creates a TIMS_Handle, which is a reference that is used by all other VI's in the driver library.

The Device index input is an integer corresponding to the index of the device on the USB bus. The default value of 0 implies the use of the first TIMS-0201 device found.

To scan the USB bus and detect the number of TIMS-0201 devices use the TIMS0201 _ListDevices_Number_Only.vi

Every application should start with this VI and conclude with a call to the TIMS Port Close.vi.

If the port has been opened, but has not been closed, another attempt to open the port will result in an error, “1, Invalid Handle”. To recover from an orphaned reference, unplug and then reconnect the USB cable.

Once the TIMS port is open, communications with the TIMS module can take place. Configuration information can be read from or written to EEPROM memory in the TIMS module and I2C or SPI communications transactions can be performed to devices connected to the TIMS module.

  - TIMS-0102 +5V Power to External Connectors

The +5V power pins on both the I2C and SPI connectors can be switched on or off using the TIMS0102_Ext_5V_Control.vi

This single VI controls power to both the I2C and SPI connectors simultaneously.

  - TIMS-0102 I2C Basic Functions

The TIMS-0102 LabVIEW drivers provide several higher-level VI’s to simplify I2C communications. These VI’s work well, for the most common forms of I2C communications. I2C scripting is also available for the more complex or obscure forms of I2C (10-bit addressing, C BUS, etc). See Section 5.3, for details on I2C scripting functionality and usage.

    - I2C Bus Configuration Functions

The I2C bus communications configuration (clock and slew rates) is stored in EEPROM on the TIMS module. This bus-timing configuration is used for all I2C communications and is retained through power cycling. Two VI’s are provided for reading or writing this configuration information.

The I2C Bus Configuration is a cluster containing:

- Target I2C Clock Rate (Khz) – DBL

- The Target I2C clock rate can be programmed to be 100KHz, 400 KHz, 1MHz, and 252 other frequencies between 39.0625 KHz and 10 MHz.

- Slew Rate – ENUM

The Slew Rate can be set to Disabled for standard speed (100K or 1M) or Enabled for high speed (400K)

    - I2C Simple I/O Functions

In addition to the configuration VIs, simple I/O VI’s are also provided.

The TIMS0102_I2C_7bit_Device_IO.vi performs a simple I2C I/O transaction to a 7-bit addressable device.

This VI can perform both a write and read operation. If the input TX DATA is empty the write operation is not performed. If the Bytes to Read input is set to 0, the read operation is not performed.

If the TX Data array input has data then this VI starts with a write operation. An I2C START bit is followed by an address byte (the R/W bit is automatically set or cleared by the firmware). The TX DATA bytes are then transmitted followed with an I2C STOP command. If the Bytes to Read input is set to a value other than zero a Read operation is performed. An I2C START command is sent followed by an address byte (with the R/W bit set to R) and then the requested number of bytes are received from the slave. The operation is completed with an I2C STOP command.

All read bytes are responded to with an ACK except for the last byte, which is responded to by a NAK.

There is no delay between the write operation and read operation. Some devices require processing time before responses can be generated (for example if you are writing to a memory location in the EEPROM you can not immediately read back the memory location without a small delay allowing the write cycle to complete). Use the I2C scripting function if you need more control over delay timing when executing commands.

A nearly identical VI, TIMS0102_I2C_7bit_Device_IO_Restart.vi also performs a simple I2C I/O transaction to a 7-bit addressable device but uses an I2C RESTART instead of the I2C STOP followed by and I2C START as in the previous VI.

The TIMS0102_I2C_7bit_Device_IO_Restart.vi performs a simple I2C I/O transaction with RESTART to a 7-bit addressable device.

This VI can perform both a write and read operation. If the input TX DATA is empty, the write operation is not performed. If the Bytes to Read input is set to 0, the read operation is not performed.

If the TX Data array input has data then this VI starts with a write operation. An I2C START bit is followed by an address byte (the R/W bit is automatically inserted). The TX DATA bytes are then transmitted followed with an I2C RESTART command if the Bytes to Read input is set to a value other than zero. A Read operation is then performed, starting with an I2C START bit followed by an address byte (with the R/W bit set to R), and then the requested number of bytes (Bytes to Read) will be received from the slave. The operation is completed with an I2C STOP command.

There is no delay between the write operation and read operation. Some devices require processing time before responses can be generated (for example if you are writing to a memory location in the EEPROM you can not immediately read back the memory location without a small delay allowing the write cycle to complete). Use the I2C scripting function if you need more control over delay timing when executing commands.

    - I2C Simple I/O Examples

Several examples that use the I2C Simple I/O VI’s can be found in the TIMS-0102 examples directories. The first example, Example_I2C_read_eeprom_memory.vi is shown below:

The Device index input is an integer corresponding to the index of the device on the USB bus. The default value of 0 implies the use of the first TIMS-0102 device found. The LabVIEW diagram is shown below.

This example VI shows the basics of I2C communications using the TIMS-0102 and the LabVIEW drivers.

The TIMS Port Open.vi and TIMS Port Close.vi are mandatory. Ports should not be left open indefinitely as some VIs return errors when connecting to open ports.

The TIMS Open Port.vi creates a reference that is used by all other VI's in the driver library. Every application should start with this VI and ensure that the TIMS Port Close.vi is also called once for every port open command.

If the port has been opened but not closed, another attempt to open the port will result in an error, “1, Invalid Handle”.

Once the TIMS port is open, communications with the TIMS module can take place and normally the I2C bus configuration is set, as in the example, to specify the desired clock and slew rates. These settings are stored in the TIMS-0102 EEPROM and are used for all I2C communications until the configuration is set again.

The +5V power pins on both the I2C and SPI connectors can be switched on or off using the TIMS0102_Ext_5V_Control.vi

This example uses the easy I/O VI, TIMS0102_7bit_Device_IO.vi, to send a command to and read a memory location from an EEPROM I2C device.

Example_I2C_write_eeprom_memory.vi is shown below.

The LabVIEW diagram is shown below.

This example VI shows the basics of I2C communications using the TIMS-0102 and the LabVIEW drivers.

The TIMS Port Open.vi and TIMS Port Close.vi are mandatory. Ports should not be left open indefinitely as some VIs return errors when connecting to open ports.

The TIMS Open Port.vi creates a reference that is used by all other VI's in the driver library. Every application should start with this VI and ensure that the TIMS Port Close.vi is also called once for every port open command.

If the port has been opened, but has not been closed, another attempt to open the port will result in an error 1, Invalid Handle.

Once the TIMS port is open, communications with the TIMS module can take place and normally the I2C bus configuration is set, as in the example, to specify the desired clock and slew rates. These settings are stored in the TIMS-0102 EEPROM and are used for all I2C communications until the configuration is set again.

The +5V power pins on both the I2C and SPI connectors can be switched on or off using the TIMS0102_Ext_5V_Control.vi

This example uses the easy I/O VI, TIMS0102_7bit_Device_IO.vi, to send a command to and write a value to a memory location in an EEPROM I2C device.

  - TIMS-0102 I2C Scripting Functions

In addition to the simple I2C I/O functions, the TIMS-0102 also allows for I2C scripting to handle the more complex or obscure communication protocol requirements. I2C scripting allows the user to build an I2C communication transaction out of the basic I2C protocol components. The script is sent to the TIMS module that then executes the script at hardware speeds. This gives you all the flexibility of using LabVIEW on a host computer to generate complex communications protocols yet ensures that the protocol is executed at the desired constant hardware generated clock rate.

This scripting mechanism allows the user to implement some other protocols not directly supported by the standard driver. C Bus and I2C 10-bit addressing are examples of functionality that can be built using the scripting functions.

    - I2C Script Building Functions

An I2C script is a string made up of a series of characters, each character representing an I2C communications protocol basic component. Script character codes are case sensitive. The script string is a simple string and standard LabVIEW string VI’s which can also be used to build the script string. The VI’s shown in the table below can also be used to assemble the script sting.

| Script Character | I2C Communication Component | LabVIEW VI |
| --- | --- | --- |
| S | START |  |
| P | STOP |  |
| R | RESTART |  |
| K | ACK |  |
| N | NAK |  |
| ? | Read Byte | continued |
| @ | Wait 1-2 msec |  |
| 00-FF | Tx Hex Data |  |

NOTE: Data sent using I2C scripting requires that the data be sent as 2 hex characters for every byte of data. All address bytes also need to be sent through the script as a 2-character hex block.

An additional VI is provided to add a 7-bit address (sent as a 2-character Hex string).

This VI generates the 2–character Hex block representing the address, with the Least significant bit (lsb) set to reflect the Read/Write bit input.

    - I2C Scripting Execution Functions

A single VI, the TIMS-0102_I2C_Script_IO.vi, provides the script download and execution functionality.

This VI takes a Tx Script transmit string input, downloads the script to the TIMS-0102, executes the script, and returns a RX Script return sting containing the data and any errors encountered in the execution of the script. Tx script strings may only be 112 characters in length.

The Rx Script return string contains the original script with extra characters inserted the contain errors or response data bytes from the slave. If an error occurs during execution of the script an “!” exclamation point is placed in the string where the error occurred. An error will terminate the I/O operation with a forced STOP condition.

    - I2C Scripting Examples

Several examples that use the I2C Scripting VI’s can be found in the TIMS-0102 examples directories. The first example, Example_I2C_script_read_eeprom_memory.vi is shown, below. This achieves the same overall functionality as the Example_I2C_read_eeprom_memory.vi that uses the simple I/O functions.

The example VI diagram shows the build up and execution of the script as well as the parsing of the Rx script string.

The lower portion of the diagram shows the building of the script. The final script sent to the script I/O is:

SA000RA1?NP

This TX script string translates to:

- S	Issue an I2C start bit

- A0	Address the device A0 for writing

- 00	Select EEPROM memory location 0 (0x00)

- R	Issue a RESTART bit

- A1	Address the device A0 for reading

- ?	Read a byte

- N	Issue a NAK bit

- P	Issue a STOP bit

As you can see, with scripting, you have much more control over how the individual communication blocks are assembled. With this flexibility, comes the extra work of building up complete I/O transactions. The simple I/O example uses at least eight fewer function calls.

A second example showing an I2C write EEMPROM memory is shown below.

The example VI diagram shows the build up and execution of the script as well as the parsing of the Rx script string.

The lower portion of the diagram shows the building of the script. The final Tx script sent to the script I/O is:

SA000FFP

This Tx script string translates to:

- S	Issue an I2C start bit

- A0	Address the device A0 for writing

- 00	Select EEPROM memory location 0 (0x00)

- FF	Hex value to write to memory location 0

- P	Issue a STOP bit

Another example script, that writes to a 27C01 EEPROM memory location, and then reads back the memory location value, is as follows.

Tx Script: SA00000P@SA000RA1?NP

Rx Script: SA00000P@SA000RA1?00NP

This Tx script string translates to:

- S	Issue a START bit

- A0	Address the device (A0)

- 00	elect EEPROM memory location 0 (ox00)

- 00	Write a zero to the memory location (0x00)

- P	Issue a STOP bit

- @	Wait 1 msec

- S	Issue a START bit

- A0	Readdress the device

- 00	Select memory location 0.

- R	Issue a RESTART bit

- A1	Readdress the device to read

- ?	Read a byte from location zero.

- N	Issue a NAK indicating no more reading

- P	Issue a STOP bit

Here is another example, which reads 16 bytes beginning at location 0x10.

Tx Script: SA000RA1?K?K?K?K?K?K?K?K?K?K?K?K?K?K?K?NP

Rx Script: SA000RA1?00K?01K?02K?03K?04K?05K?06K?07K?08K?09K?0A?0B?0C?0D?0E?0FNP

  - TIMS-0102 SPI Functions

The TIMS-0102 LabVIEW drivers provide several VI’s to simplify SPI communications.

    - SPI Device Configuration Functions

Unlike I2C, individual SPI devices operating on the same bus can all be operating at different clock rates and in different clock edge configurations. Because of this difference, the TIMS-0102 allows up to 16 different SPI device configurations to be stored in EEPROM (compared to just one setting used for all I2C devices). You only need to send an SPI device configuration once and it will be used for all subsequent operations directed at that particular SPI device. The TIMS-0102 will automatically switch to pre-configured settings stored for the particular SPI device addressed so that you don’t have to keep sending clock configuration settings every time you address a different device.

The SPI device configurations are retained through power cycling. Two VI’s are provided for reading or writing this configuration information.

The SPI Device Ref input must be an integer number between 1 and 16.

The SPI Device Configuration is a cluster containing:

- Tx Clocking – ENUM

    - Clock Idle Low, TX Falling Edge

    - Clock Idle High, TX Falling Edge

    - Clock Idle Low, TX Rising Edge

    - Clock Idle High, TX Rising Edge

- Rx Clocking – ENUM

    - Rx Sampled at Middle of TX Output Period

    - Rx Sampled at End of TX Output Period

- Target Clock Rate – DBL Float

    - Target clock rate must be in range from 39.0625 KHz to 10 MHz.

The second VI is for reading a SPI device configuration.

    - SPI Device Select

The TIMS-0102 comes with eight programmable device or chip select lines that are available at the output SPI DB-15F connector. These chip select lines can be independently selected or used to generate a combination of a 4-bit address and single chip select line.

The SPI Device Select is an ENUM (enumerated constant) with values of:

- ADR x0, nCS4

- ADR x1, nCS4

- ADR x2, nCS4

- ADR x3, nCS4

- ADR x4, nCS4

- ADR x5, nCS4

- ADR x6, nCS4

- ADR x7, nCS4

- ADR x8, nCS4

- ADR x9, nCS4

- ADR xA, nCS4

- ADR xB, nCS4

- ADR xC, nCS4

- ADR xD, nCS4

- ADR xE, nCS4

- ADR xF, nCS4

- nCS0

- nCS1

- nCS2

- nCS3

- nCS4

- nCS5

- nCS6

- nCS7

- No Select

When ADRx0, nCS4 through ADRxF, nCS4 is selected the corresponding SPI Device configuration is recalled and used in the following transaction. The address lines are updated with the requested address and then the nCS4 (i.e., nEN Not Enable) is set low.

If nCS0 through nCS7 is selected then SPI Device Configurations 0 through 7 is recalled and used in the following transaction. The requested line, nCS0 – nCS7, is set low.

Selecting the No Select option sets all line high.

    - SPI I/O Functions

The TIMS-0102_SPI_Data_IO.vi performs a basic SPI communication transaction. This VI does not affect the device select lines; they must be separately enabled.

This VI is intended to be used when more than 112 Bytes need to be read or sent out between chip select enabled and chip select disabled, as in the following VI.

A second SPI VI, the TIMS0102_SPI_Device_IO.vi can also be used for performing a basic SPI communications transaction.

The TIMS0102_SPI_Device_IO.vi performs the chip select operation, a data I/O operations up to 112 bytes, then a chip deselect operation.

    - SPI Examples

Several examples that use the SPI I/O Function VI’s can be found in the TIMS-0102 examples directories The first example, Example_SPI_read_eeprom_memory.vi is shown below:

The diagram for this VI is shown below.

This example VI shows the basics of SPI communications using the TIMS-0102 and the LabVIEW drivers.

The TIMS Port Open.vi and TIMS Port Close.vi are mandatory and Ports should not be left open indefinitely as some VI's cannot execute on open ports.

The TIMS Open Port.vi creates reference that is used by all other VI's in the driver library. Every application should start with this VI and ensure that the TIMS Port Close.vi is also called once for every port open command.

If the port has been opened and not closed, another attempt to open the port will result in an error 1, Invalid Handle.

Once the TIMS port is open, communications with the TIMS module can take place and normally SPI Device configurations are set, as in the example, to specify the desired clock rates and edge settings. These settings are stored in the TIMS-0102 EEPROM and are used for all I2C communications until the configuration is set again.

The +5V power pins on both the I2C and SPI connectors can be switched on or off using the TIMS0102_Ext_5V_Control.vi

  - TIMS-0102 General Functions

The VI’s in the General Functions section are related to TIMS-0102 module functions not specifically I2C or SPI. These functions include opening and closing a communications port to the TIMS-0102 module, scanning the USB bus for TIMS modules, and reading configuration information from an a TIMS-0102 EEPROM.

The TIMS-0102 Port Open and Close VI’s are described above in section 4.1 and the TIMS-0102 +5V Power.vi is described in section 4.2. It is repeated here for convenience.

The driver VI’s TIMS Port Open.vi and TIMS Port Close.vi are used for opening and closing communications ports to the TIMS-0102 module.

The TIMS Open Port.vi creates a TIMS0102_Handle, which is a reference that is used by all other VI's in the driver library.

The Device index input is an integer corresponding to the index of the device on the USB bus. The default value of 0 implies the use of the first TIMS-0102 device found.

Every application should start with this VI and conclude with a call to the TIMS Port Close.vi.

To scan the USB bus and detect the number of attached TIMS-0102 devices, use the TIMS0102_ListDevices_Number_Only.vi

If the port has been opened, but is not closed, another attempt to open the port will result in an error 1, Invalid Handle.

Once the TIMS port is open, communications with the TIMS module can take place. Configuration information can be read from or written to EEPROM memory in the TIMS module and I2C or SPI communications transactions can be performed to devices connected to the TIMS module.

The +5V power pins on both the I2C and SPI connectors can be switched on or off using the TIMS0102_Ext_5V_Control.vi

This single VI controls power to both the I2C and SPI connectors simultaneously.

The TIMS0102_Device_Reset.vi resets the microprocessor in the TIMS-0102 module.

The TIMS-0102 module can save a user-defined name in EEPROM to make future identification easier.

To read back the User ID, use the TIMS0102_Device_User_ID_Get.vi.

The TIMS0102_Device_Status.vi returns 24 bits of status information, 16 from the microprocessor and 8 from the firmware. (This information can be used by technical support in troubleshooting problems.)

The Device Descriptor cluster contains the TIMS model number, serial number, user ID and function description (stored in EEPROM).

For even more detailed configuration information stored during the manufacture of the TIMS-0102 module, use the TIMS0102_Device_ID.vi (stored in EEPROM).

This VI returns assembly numbers and revisions of components within the TIMS-0102 module and contains version information about the firmware.

- Warranty

WireWorks West, Inc. warrants its products for a period of one year from date of delivery against defects in material or workmanship. Returned product will either be repaired or replaced at the option of WireWorks West, Inc.

Specifications are subject to change without notice.

- About WireWorks West, Inc.

Founded in 1998, WireWorks West, Inc. provides software and services for industrial automation and interactive data networking. WireWorks West, Inc. focuses on industry-specific solutions for Pharmaceuticals and Biotechnology, Electronics Manufacturing, Networking, and Telecommunications.

WireWorks West, Inc. is a privately held company with headquarters in San Francisco, CA.

- Contacting wireworks west, inc.

To find out more about the TIMS-0102, please contact us at: 1-800-755-1400

For information on our products and services, please visit our website at: www.wireworkswest.com.

For technical support issues, please contact us at 1-415-348-1408

# Appendix A

## LabVIEW Function Reference

TIMS0102_Device_Descriptor.vi

Returns the four basic TIMS-0102 module identification parameters. Model number, serial number, function and user ID are returned.

Connector Pane

Controls and Indicators

|  | TO Device |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | Device Descriptor |  |
|  |  | Model No. |
|  |  | Serial No. |
|  |  | Function |
|  |  | User ID |
|  | ref out TIMS Device Handle |  |

TIMS0102_Device_Reset.vi

Resets the TIMS-0102 processor.

Connector Pane

Controls and Indicators

|  | TO Device Specifies which processor to address on a TIMS device. Default is D0, the master processor. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_Device_Status.vi

Returns the TIMS-0102 Device Status clusters. 24 bits of status information is available describing the state of USB communications and the TIMS-0102 processor.

Connector Pane

Controls and Indicators

|  | TO Device Specifies which processor to address on a TIMS device. Default is D0, the master processor. |  |  |
| --- | --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |  |
|  | ref in TIMS Device Handle |  |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  | Device Status |  |  |
|  |  | RCON |  |
|  |  |  | BOR* |
|  |  |  | POR* |
|  |  |  | PD* |
|  |  |  | TO* |
|  |  |  | RI* |
|  |  |  | (na5) |
|  |  |  | (na6) |
|  |  |  | IPEN |
|  |  | STKPTR |  |
|  |  |  | SP0 |
|  |  |  | SP1 |
|  |  |  | SP2 |
|  |  |  | SP3 |
|  |  |  | SP4 |
|  |  |  | (na5) |
|  |  |  | STKUNF |
|  |  |  | STKFUL |
|  |  | Misc |  |
|  |  |  | FLASH Prgm |
|  |  |  | SW Valid |
|  |  |  | SWDTEN |
|  |  |  | Framing |
|  |  |  | Overrun |
|  |  |  | (na5) |
|  |  |  | Mfg Mode |
|  |  |  | Reset |
|  | ref out TIMS Device Handle |  |  |

TIMS0102_Device_User_ID.vi

Returns the user defined TIMS-0102 module identification string.

Connector Pane

Controls and Indicators

|  | TO Device |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | User ID |  |
|  | ref out TIMS Device Handle |  |

TIMS0102_Device_User_ID_Set.vi

Writes the User ID to the TIMS-0102 EEPROM.

Connector Pane

Controls and Indicators

|  | TO Device Specifies which processor to address on a TIMS device. Default is D0, the master processor. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | User ID The user supplied device ID |  |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_Ext_5V_Control.vi

Controlls power to the +5V pins of the I2C and SPI connectors on the TIMS-0102 module.

Connector Pane

Controls and Indicators

|  | Ext 5V ON Selects the state of the 5V output line. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_ListDevices_Number_Only.vi

Returns the number of TIMS modules found on the USB bus.

Connector Pane

Controls and Indicators

|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
| --- | --- | --- |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | No_of_Devices Returns number of FTDI devices connected to the PC. TIMS-0102 devices use the FTDI USB driver. It is possible that there will be more than just TIME-0102 devices using this driver. |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |

TIMS0102_Unit_ID.vi

Returns TIMS-0102 module information stored in EEPROM including device name and descriptions, Part and serial numbers, and a user defined ID.

Connector Pane

Controls and Indicators

|  | TO Device Specifies which processor to address on a TIMS device. Default is D0, the master processor. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | ref in TIMS Device Handle |  |
|  | User ID The user supplied ID of the device. |  |
|  | Model ID |  |
|  |  | Model No. |
|  |  | Description 1 |
|  |  | Description 2 |
|  | PCB Assy ID |  |
|  |  | Part No. |
|  |  | Serial No. |
|  |  | Misc Info |
|  | Device ID |  |
|  |  | Device Function |
|  |  | Device Description |
|  |  | Application SW ID |
|  |  | Monitor SW ID |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | TOP Assy ID |  |
|  |  | Part No. |
|  |  | Serial No. |
|  |  | Misc Info |
|  | ref out TIMS Device Handle |  |

TIMS_Port_Close.vi

Closes a TIMS module reference (TIMS0102_Handle).

Connector Pane

Controls and Indicators

|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
| --- | --- | --- |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |

TIMS_Port_Open.vi

Returns a reference (TIMS0102_Handle) to the TIMS module at Device Index on the USB bus.

Connector Pane

Controls and Indicators

|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
| --- | --- | --- |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | Device Index Index of the USB device to open. Indexes begin at 0. |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_I2C_7bit_Device_IO.vi

Performs a simple I2C I/O transaction to a 7-bit addressable device.

This VI can perform both a write and read operation. If the input TX DATA is empty the write operation is not performed. If the Bytes to Read input is set to 0 the read operation is not performed.

If the TX Data array input has data then this VI starts with a write operation. An I2C START bit is followed by an address byte (the R/W bit is automatically inserted). The TX DATA bytes are then transmitted followed with an I2C STOP command. If the Bytes to Read input is set to a value other than zero a Read operation is performed. An I2C START command is sent followed by an address byte (with the R/W bit set to R) and then the requested number of bytes are received from the slave. The operation is completed with an I2C STOP command.

There is no delay between the write operation and read operation. Some devices require processing time before responses can be generated (for example if you are writing to a memory location in the EEPROM you can not immediately read back the memory location without a small delay allowing the write cycle to complete). Use the I2C scripting function if you need more control over delay timing when executing commands.

Connector Pane

Controls and Indicators

|  | TX DATA Array of U8 bytes to be sent to the I2C device. |  |
| --- | --- | --- |
|  |  | FROM |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | Bytes to Read Number of bytes to be read from the I2C device. |  |
|  | I2C Device 7-Bit Address The 7bit address of the I2C slave device. |  |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | RX DATA Array of U8 bytes read from the I2C device. |  |
|  |  | FROM |
|  | ref out TIMS Device Handle |  |

TIMS0102_I2C_7bit_IO_Restart.vi

Performs a simple I2C I/O transaction with RESTART to a 7-bit addressable device.

This VI can perform both a write and read operation. If the input TX DATA is empty the write operation is not performed. If the Bytes to Read input is set to 0 the read operation is not performed.

If the TX Data array input has data then this VI starts with a write operation. An I2C START bit is followed by an address byte (the R/W bit is automatically inserted). The TX DATA bytes are then transmitted followed with an I2C RESTART command if the Bytes to Read input is set to a value other than zero.  A Read operation is then performed starting with an I2C START bit followed by an address byte (with the R/W bit set to R) and then the requested number of bytes are received from the slave. The operation is completed with an I2C STOP command.

There is no delay between the write operation and read operation. Some devices require processing time before responses can be generated (for example if you are writing to a memory location in the EEPROM you can not immediately read back the memory location without a small delay allowing the write cycle to complete). Use the I2C scripting function if you need more control over delay timing when executing commands.

Connector Pane

Controls and Indicators

|  | TX DATA Array of bytes to send to the slave I2C device. |  |
| --- | --- | --- |
|  |  | FROM |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | Bytes to Read Number of bytes to read from the slave device. |  |
|  | I2C Device 7-Bit Address Address of the I2C slave device. |  |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | RX DATA Array of U8 bytes read from the I2C device. |  |
|  |  | FROM |
|  | ref out TIMS Device Handle |  |

TIMS0102_I2C_Bus_Config_Get.vi

Returns the I2C Bus Configuration from the TIMS-0102 EEPROM.

The TIMS-0102 stores I2C bus configuration information in EEPROM.

The I2C Bus Configuration includes:

Target Clock Rate: The I2C clock rate is programmable to 100KHz, 400KHz, 1MHz, and 252 other frequencies between 39.0625 KHz and 10MHz.

Slew Rate: The Slew Rate can be set to Disables for standard speed (100K or 1M) or Enabled for high speed (400K).

Connector Pane

Controls and Indicators

|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
| --- | --- | --- |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | I2C Bus Configuration |  |
|  |  | Slew Rate |
|  |  | Target I2C Clock Rate (Khz) |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_I2C_Bus_Config_Set.vi

Writes the I2C Bus Configuration parameters to the TIMS-0102 EEPROM.

The TIMS-0102 stores I2C bus configuration information in EEPROM.

The I2C Bus Configuration includes:

Target Clock Rate: The I2C clock rate should be set to a value compatible with all devices on your I2C bus. The I2C clock rate is programmable to 100KHz, 400KHz, 1MHz, and 252 other frequencies between 39.0625 KHz and 10MHz.

Slew Rate: The Slew Rate can be set to Disables for standard speed (100K or 1M) or Enabled for high speed (400K).

Connector Pane

Controls and Indicators

|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
| --- | --- | --- |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | I2C Bus Configuration |  |
|  |  | Slew Rate |
|  |  | Target I2C Clock Rate (Khz) |
|  | ref in TIMS Device Handle |  |
|  | Best I2C Clock Rate (Khz) |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_I2C_Script_7bit_Address.vi

Adds an I2C ADDRESS BYTE script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Address (7-bit) |
|  | Read/Write bit |
|  | Script output |

TIMS0102_I2C_Script_ACK.vi

Adds an I2C ACK script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Script output |

TIMS0102_I2C_Script_IO.vi

Sends an I2C script to the TIMS-0102 module then executes the script and returns any bytes read during the execution of the script.

An I2C script is a string made up of a series of characters, each character representing an I2C communications protocol basic component. Script character codes are case sensitive.

S - Start

P - Stop

R - Restart

K - ACK

N - NAK

? - Read Byte

@ - Wait 1-2 msec, minimum of 1 msec, not more than 2 msec

0-F - Tx Hex character, must be in pairs

An example script to write to a 27C01 EEPROM memory location and then read back the memory location value is as follows.

Tx Script:  SA00000P@SA000RA1?NP

Rx Script:  SA00000P@SA000RA1?00NP

This translates to:

1. Issue a START bit

2. Address the device (A0)

3. Select EEPROM memory location 0 (ox00)

4. Write a zero to the memory location (0x00)

5. Issue a STOP bit

6. Wait 1 msec

7. Issue a START bit

8. Readdress the device and select memory location 0.

9. Issue a RESTART bit

10. Readdress the device to read

11. Read a byte from location zero.

12. Issue a NAK indicating no more reading

13. Issue a STOP bit

The I2C scripting functionality allows users to build up a series of I2C communications elements, send them in mass to the TIMS-0102 processor and then have the clocked out consistently at the configured clock rate.

This scripting mechanism allows the user to implement some other protocols not directly supported by the standard driver. C Bus and I2C 10-bit addressing are examples of functionality that can be built using the scripting function.

The Rx Script return string contains the original script with extra characters inserted the contain errors or response data bytes from the slave. If an error occurs a "!" exclamation point is placed in the string where the error occurred.

Another example that reads 16 bytes beginning at location 0x10:

Tx Script: SA000RA1?K?K?K?K?K?K?K?K?K?K?K?K?K?K?K?NP

Rx Script: SA000RA1?00K?01K?02K?03K?04K?05K?06K?07K?08K?09K?0A?0B?0C?0D?0E?0FNP

Connector Pane

Controls and Indicators

|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | TX Script |  |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | RX Script |  |
|  | I2C Subroutine Error |  |
|  | ref out TIMS Device Handle |  |

TIMS0102_I2C_Script_NAK.vi

Adds an I2C NAK script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Script output |

TIMS0102_I2C_Script_Read_Byte.vi

Adds a READ BYTE script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Script output |

TIMS0102_I2C_Script_Restart.vi

Adds an I2C RESTART command script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Script output |

TIMS0102_I2C_Script_Start.vi

Adds an I2C START script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Script output |

TIMS0102_I2C_Script_Stop.vi

Adds an I2C STOP script element to the script string..

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Script output |

TIMS0102_I2C_Script_Tx_Byte.vi

Adds DATA BYTE script elements to the script string.

Connector Pane

Controls and Indicators

|  | Script input |  |
| --- | --- | --- |
|  | Data Bytes |  |
|  |  | data byte |
|  | Script output |  |

TIMS0102_I2C_Script_Wait.vi

Adds a WAIT script element to the script string.

Connector Pane

Controls and Indicators

|  | Script input |
| --- | --- |
|  | Number of waits (1-2msec per wait) |
|  | Script output |

TIMS0102_SPI_Data_IO.vi

Performs an SPI I/O transaction without changing the state of any device enable line.

Connector Pane

Controls and Indicators

|  | TX DATA |  |
| --- | --- | --- |
|  |  | FROM |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | RX DATA |  |
|  |  | FROM |
|  | ref out TIMS Device Handle |  |

TIMS0102_SPI_Device_Config_Get.vi

Returns the SPI Device Configuration from the TIMS-0102 EEPROM.

The TIMS-0102 can store up to 16 different SPI device configurations in EEPROM. This allows you to pre-configure communications with multiple SPI devices, each using different clock rates and clock edge configurations.

The SPI Device Configuration includes:

Tx Clocking: The clock idle state can be configured to be high or low and transmission can be configured to be on the falling or rising edge.

Rx Clocking: Rx Clocking can be configured so that when clocking in data from the slave, data is sampled at either the Middle or End of the TX output period.

Target Clock Rate: The TIMS-0102 supports 255 clock speeds from 39.0625 KHz to 10 MHz, including standard clock rates of 100 KHz, 400 KHz, and 1 MHz.

Connector Pane

Controls and Indicators

|  | SPI Device Ref Specifies which of the 16 available device configurations to retrieve. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | Device Configuration |  |
|  |  | TX Clocking |
|  |  | RX Clocking |
|  |  | Target SPI Clock Frequency (KHz) |
|  | ref out TIMS Device Handle |  |

TIMS0102_SPI_Device_Config_Set.vi

Writes the SPI Device Configuration to the TIMS-0102 EEPROM.

The TIMS-0102 can store up to 16 different SPI device configurations in EEPROM. This allows you to pre-configure communications with multiple SPI devices, each using different clock rates and clock edge configurations. When issuing SPI I/O commands you can reference the preconfigured settings and quickly switch between communications with devices on the same bus but using different clock rates and edge processing configurations. This saves quite a bit of time required to constantly process the reconfigure commands.

The SPI Device Configuration includes:

Tx Clocking: The clock idle state can be configured to be high or low and transmission can be configured to be on the falling or rising edge.

Rx Clocking: Rx Clocking can be configured so that when clocking in data from the slave, data is sampled at either the Middle or End of the TX output period.

Target Clock Rate: The TIMS-0102 supports 255 clock speeds from 39.0625 KHz to 10 MHz, including standard clock rates of 100 KHz, 400 KHz, and 1 MHz.

Connector Pane

Controls and Indicators

|  | Device Configuration |  |
| --- | --- | --- |
|  |  | TX Clocking |
|  |  | RX Clocking |
|  |  | Target SPI Clock Frequency (KHz) |
|  | SPI Device Ref Specifies which of the 16 available device configurations to set. |  |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | Best Freq (kHz) |  |
|  | ref out TIMS Device Handle |  |

TIMS0102_SPI_Device_Enable.vi

Enables a chip select line.

The following options are available:

ADR x0, nCS4

ADR x1, nCS4

ADR x2, nCS4

ADR x3, nCS4

ADR x4, nCS4

ADR x5, nCS4

ADR x6, nCS4

ADR x7, nCS4

ADR x8, nCS4

ADR x9, nCS4

ADR xA, nCS4

ADR xB, nCS4

ADR xC, nCS4

ADR xD, nCS4

ADR xE, nCS4

ADR xF, nCS4

nCS0

nCS1

nCS2

nCS3

nCS4

nCS5

nCS6

nCS7

No Select

Connector Pane

Controls and Indicators

|  | SPI Device Select Specifies which SPI device to address. When communicating with an SPI device, the TIMS-0102 will use the config. settings corresponding to the address you provide here. |  |
| --- | --- | --- |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | ref out TIMS Device Handle |  |

TIMS0102_SPI_Device_IO.vi

Performs an SPI I/O transaction after first setting the state of the specified device enable line.

Connector Pane

Controls and Indicators

|  | TX DATA |  |
| --- | --- | --- |
|  |  | FROM |
|  | timeout (ms) port number varies by platform. The parameters for serial port numbers depend on the whether you use Macintosh, Windows, or Unix. |  |
|  | error in (no error) The error in cluster can accept error information wired from VIs previously called. Use this information to decide if any functionality should be bypassed in the event of errors from other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | SPI Device Select Specifies which SPI device to address. When communicating with an SPI device, the TIMS-0102 will use the config. settings corresponding to the address you provide here. |  |
|  | ref in TIMS Device Handle |  |
|  | error out The error out cluster passes error or warning information out of a VI to be used by other VIs. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |  |
|  |  | status The status Boolean is either TRUE (X) for an error, or FALSE (checkmark) for no error or a warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | code The code input identifies the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  |  | source The source string describes the origin of the error or warning. The pop-up option Explain Error (or Explain Warning) gives more information about the error displayed. |
|  | RX DATA |  |
|  |  | FROM |
|  | ref out TIMS Device Handle |  |

## TIMS-0102 USB – I2C/SPI Interface Error Codes

| Error Name | Type | Firmware Error Code | FTDI Error Code | LabVIEW Driver Error Code |
| --- | --- | --- | --- | --- |
| Status_OK | STATUS_ALL | 0x00 |  | 0 |
| Status_None | STATUS_ALL | 0x00 |  | 0 |
| Status_Unknown_Error | STATUS_ALL | 0xF0 |  | 6003 |
| Status_Device_Open_Error | STATUS_ALL | 0xF1 |  | 6004 |
| Status_Device_Timeout_Error | STATUS_ALL | 0xF2 |  | 6005 |
| Status_RX_Buffer_Full | STATUS_ALL | 0xF3 |  | 6006 |
| Status_RX_LRC_Error | STATUS_ALL | 0xF4 |  | 6007 |
| Status_TX_LRC_Error | STATUS_ALL | 0xF5 |  | 6008 |
| Status_Unknown_Type | STATUS_ALL | 0xF6 |  | 6009 |
| Status_Bad_Data_Length | STATUS_ALL | 0xF7 |  | 6010 |
| Status_Bad_Data_Request_Length | STATUS_ALL | 0xF8 |  | 6011 |
| Status_Function_Timeout_Error | STATUS_ALL | 0xF9 |  | 6012 |
| Status_I2Cm_START_Cond_Error | I2C_SUB_Error | 0x10 |  | 6013 |
| Status_I2Cm_START_BCOL_Error | I2C_SUB_Error | 0x11 |  | 6014 |
| Status_I2Cm_START_TO_Error | I2C_SUB_Error | 0x12 |  | 6015 |
| Status_I2Cm_START_Ver_Error | I2C_SUB_Error | 0x13 |  | 6016 |
| Status_I2Cm_RESTART_Cond_Error | I2C_SUB_Error | 0x20 |  | 6017 |
| Status_I2Cm_RESTART_BCOL_Error | I2C_SUB_Error | 0x21 |  | 6018 |
| Status_I2Cm_RESTART_TO_Error | I2C_SUB_Error | 0x22 |  | 6019 |
| Status_I2Cm_RESTART_Ver_Error | I2C_SUB_Error | 0x23 |  | 6020 |
| Status_I2Cm_WR_BYTE_Cond_Error | I2C_SUB_Error | 0x30 |  | 6021 |
| Status_I2Cm_WR_BYTE_BCOL_Error | I2C_SUB_Error | 0x31 |  | 6022 |
| Status_I2Cm_WR_BYTE_TO_Error | I2C_SUB_Error | 0x32 |  | 6023 |
| Status_I2Cm_RD_ACK_BCOL_Error | I2C_SUB_Error | 0x41 |  | 6024 |
| Status_I2Cm_RD_ACK_TO_Error | I2C_SUB_Error | 0x42 |  | 6025 |
| Status_I2Cm_RD_BYTE_BCOL_Error | I2C_SUB_Error | 0x51 |  | 6026 |
| Status_I2Cm_RD_BYTE_TO_Error | I2C_SUB_Error | 0x52 |  | 6027 |
| Status_I2Cm_WR_ACK_BCOL_Error | I2C_SUB_Error | 0x61 |  | 6028 |
| Status_I2Cm_WR_ACK_TO_Error | I2C_SUB_Error | 0x62 |  | 6029 |
| Status_I2Cm_WR_NAK_BCOL_Error | I2C_SUB_Error | 0x71 |  | 6030 |
| Status_I2Cm_WR_NAK_TO_Error | I2C_SUB_Error | 0x72 |  | 6031 |
| Status_I2Cm_STOP_BCOL_Error | I2C_SUB_Error | 0x81 |  | 6032 |
| Status_I2Cm_STOP_TO_Error | I2C_SUB_Error | 0x82 |  | 6033 |
| Status_I2Cm_STOP_Ver_Error | I2C_SUB_Error | 0x83 |  | 6034 |
| CT0102D0_Ext_5V_ON | TIMS-0102 | 0x10 |  | 6035 |
| CT0102D0_Ext_5V_OFF | TIMS-0102 | 0x11 |  | 6036 |
| CT0102D0_SPI_Device_Config_Set | TIMS-0102 | 0x20 |  | 6037 |
| CT0102D0_SPI_Device_Config_Get | TIMS-0102 | 0x21 |  | 6038 |
| CT0102D0_SPI_Device_Select | TIMS-0102 | 0x22 |  | 6039 |
| CT0102D0_SPI_Data_IO | TIMS-0102 | 0x23 |  | 6040 |
| CT0102D0_SPI_Device_IO | TIMS-0102 | 0x24 |  | 6041 |
| CT0102D0_I2C_Config_Set | TIMS-0102 | 0x40 |  | 6042 |
| CT0102D0_I2C_Config_Get | TIMS-0102 | 0x41 |  | 6043 |
| CT0102D0_I2C_Device_IO_A7 | TIMS-0102 | 0x42 |  | 6044 |
| CT0102D0_I2C_Device_IO_A7R | TIMS-0102 | 0x43 |  | 6045 |
| CT0102D0_I2C_Script_IO | TIMS-0102 | 0x44 |  | 6046 |
| ST0102D0_Error_I2C_Address | TIMS-0102 | 0x90 |  | 6047 |
| ST0102D0_Error_I2C_SUB_Error | TIMS-0102 | 0x91 |  | 6048 |
| Control_Get_Monitor_SW_ID | MONITOR_USER | 0xE0 |  | 6049 |
| Control_Get_Device_SW_ID | MONITOR_USER | 0xE1 |  | 6050 |
| Control_Get_Device_ID | MONITOR_USER | 0xE2 |  | 6051 |
| Control_Get_PCB_Assy_ID | MONITOR_USER | 0xE3 |  | 6052 |
| Control_Get_TOP_Assy_ID | MONITOR_USER | 0xE4 |  | 6053 |
| Control_Get_Model_ID | MONITOR_USER | 0xE5 |  | 6054 |
| Control_Get_Model_Info | MONITOR_USER | 0xE6 |  | 6055 |
| Control_Get_Device_User_ID | MONITOR_USER | 0xE7 |  | 6056 |
| Control_Set_Device_User_ID | MONITOR_USER | 0xE8 |  | 6057 |
| Control_Get_Msec_Clock | MONITOR_USER | 0xE9 |  | 6058 |
| Control_Set_Msec_Clock | MONITOR_USER | 0xEA |  | 6059 |
| Control_Get_Device_Descriptor | MONITOR_USER | 0xED |  | 6060 |
| Control_Get_Device_Status | MONITOR_USER | 0xEE |  | 6061 |
| Control_Device_Reset | MONITOR_USER | 0xEF |  | 6062 |
| Control_RAM_Get_Bytes | MONITOR_MFG | 0xF0 |  | 6063 |
| Control_RAM_Set_Bytes | MONITOR_MFG | 0xF1 |  | 6064 |
| Control_EE_Get_Bytes | MONITOR_MFG | 0xF2 |  | 6065 |
| Control_EE_Set_Bytes | MONITOR_MFG | 0xF3 |  | 6066 |
| Control_FLASH_Get_Bytes | MONITOR_MFG | 0xF4 |  | 6067 |
| Control_FLASH_Block_Erase | MONITOR_MFG | 0xF5 |  | 6068 |
| Control_FLASH_Block_Write | MONITOR_MFG | 0xF6 |  | 6069 |
| Status_FLASH_Invalid_Address | MONITOR_MFG | 0x81 |  | 6070 |
| Status_FLASH_Block_Erase_Error | MONITOR_MFG | 0x82 |  | 6071 |
| Status_FLASH_Block_Write_Error | MONITOR_MFG | 0x83 |  | 6072 |
| Control_FLASH_Clr_Prgm_Flag | MONITOR_MFG | 0xF7 |  | 6073 |
| Control_Set_Device_ID | MONITOR_MFG | 0xF8 |  | 6074 |
| Control_Set_PCB_Assy_ID | MONITOR_MFG | 0xF9 |  | 6075 |
| Control_Set_TOP_Assy_ID | MONITOR_MFG | 0xFA |  | 6076 |
| Control_Set_Model_ID | MONITOR_MFG | 0xFB |  | 6077 |
| Control_Set_Model_Info | MONITOR_MFG | 0xFC |  | 6078 |
| Control_TREF | MONITOR_MFG | 0xFE |  | 6079 |
| Control_Mfg_Mode | MONITOR_MFG | 0xFF |  | 6080 |
| Status_Enter_Mfg_Mode_Error | MONITOR_MFG | 0xFF |  | 6081 |
| FT_OK | FTDI | 0 |  | 6082 |
| FT_INVALID_HANDLE | FTDI | 1 |  | 6083 |
| FT_DEVICE_NOT_FOUND | FTDI | 2 |  | 6084 |
| FT_DEVICE_NOT_OPENED | FTDI | 3 |  | 6085 |
| FT_IO_ERROR | FTDI | 4 |  | 6086 |
| FT_INSUFFICIENT_RESOURCES | FTDI | 5 |  | 6087 |
| FT_INVALID_PARAMETER | FTDI | 6 |  | 6088 |
| FT_INVALID_BAUD_RATE | FTDI | 7 |  | 6089 |
| FT_DEVICE_NOT_OPENED_FOR_ERASE | FTDI | 8 |  | 6090 |
| FT_DEVICE_NOT_OPENED_FOR_WRITE | FTDI | 9 |  | 6091 |
| FT_FAILED_TO_WRITE_DEVICE | FTDI | 10 |  | 6092 |
| FT_EEPROM_READ_FAILED | FTDI | 11 |  | 6093 |
| FT_EEPROM_WRITE_FAILED | FTDI | 12 |  | 6094 |
| FT_EEPROM_ERASE_FAILED | FTDI | 13 |  | 6095 |
| FT_EEPROM_NOT_PRESENT | FTDI | 14 |  | 6096 |
| FT_EEPROM_NOT_PROGRAMMED | FTDI | 15 |  | 6097 |
| FT_INVALID_ARGS | FTDI | 16 |  | 6098 |
| FT_OTHER_ERROR | FTDI | 17 |  | 6099 |

Table of Contents

1.	TIMS-0102 I2C and SPI Interface Module Hardware	3

1.1	Connector Details	5

1.1.1	I2C Connector	Error! Bookmark not defined.

1.1.1.1	Mating Connectors	Error! Bookmark not defined.

1.1.2	SPI Connector	Error! Bookmark not defined.

1.1.2.1	Mating Connector: DB-15P (Male) Amp	Error! Bookmark not defined.

1.1.3	USB Connector	7

1.2	I2C Signals	Error! Bookmark not defined.

1.2.1	Voltage Level Buffering	Error! Bookmark not defined.

1.2.2	Pull-up Resistors	Error! Bookmark not defined.

1.2.3	Drive Current	Error! Bookmark not defined.

1.2.4	Clock rates	Error! Bookmark not defined.

1.2.5	Addressing	Error! Bookmark not defined.

1.2.6	Known Limitations	Error! Bookmark not defined.

1.3	SPI Signals	Error! Bookmark not defined.

1.3.1	Transmit (Tx) clocking	Error! Bookmark not defined.

1.3.2	Receive (Rx) clocking	Error! Bookmark not defined.

1.3.3	Drive Current	Error! Bookmark not defined.

1.3.4	Clock Rates and Speed	Error! Bookmark not defined.

1.3.4.1	Range 1.	Error! Bookmark not defined.

1.3.4.2	Range 2.	Error! Bookmark not defined.

1.3.4.3	Range 3.	Error! Bookmark not defined.

1.3.5	Chip-Select	Error! Bookmark not defined.

1.3.6	Microwire Compatibility	Error! Bookmark not defined.

1.3.7	Known Limitations	Error! Bookmark not defined.

1.4	External +Vref	Error! Bookmark not defined.

2.	General Software Description	8

3.	Software Installation	10

3.1	Preparing For Installation	10

3.2	Installation	11

3.2.1	Installing the TIMS-0102 USB Drivers	11

3.2.2	Verifying Installation	14

3.2.3	Installing the TIMS-0102 LabVIEW Drivers and Code	14

4.	TIMS-0102 LabVIEW Instrument Drivers	15

4.1	TIMS-0102 Communications Port	16

4.2	TIMS-0102 +5V Power to External Connectors	17

4.3	TIMS-0102 I2C Basic Functions	17

4.3.1	I2C Bus Configuration Functions	17

4.3.2	I2C Simple I/O Functions	18

4.3.3	I2C Simple I/O Examples	19

4.4	TIMS-0102 I2C Scripting Functions	21

4.4.1	I2C Script Building Functions	21

4.4.2	I2C Scripting Execution Functions	22

4.4.3	I2C Scripting Examples	22

4.5	TIMS-0102 SPI Functions	26

4.5.1	SPI Device Configuration Functions	26

4.5.2	SPI Device Select	27

4.5.3	SPI I/O Functions	28

4.5.4	SPI Examples	29

TIMS-0102 General Functions	30

5.	Warranty	33

6.	About WireWorks West, Inc.	33

7.	Contacting wireworks west, inc.	33

Appendix	34

LabVIEW Function Reference And ERROR CODES	34